| COMPETENCY BASED LEARNING MATERIAL |  |  |
| --- | --- | --- |
| Sector: ###### AUTOMOTIVE/LAND TRANSPORT SECTOR |  |  |  |
| ###### Distinctive Area of Competence and Qualifications: Automotive Servicing NC I |  |  |  |
| ###### Unit of Competency: Perform Periodic Maintenance of Steering System |  |  |  |
| Module Title: Performing Periodic Maintenance of Suspension System |  |  |  |
|  | AUTOHUB INSTITUTE OF TECHNOLOGY |  |  |

HOW TO USE THIS COMPETENCY BASED LEARNING MODULE

Welcome to the Module:  Performing Periodic Maintenance of Suspension System. This module contains training materials and activities for you to complete.

The unit of competency Perform Periodic Maintenance of Suspension System contains the knowledge, skills and attitudes required for Automotive Servicing NC I . It is one of the Core Modules at   National Certificate Level I (NC I ).

You are required to go through a series of learning activities in order to complete each learning outcome of the module.  In each learning outcome there are Information Sheets, Resource Sheets and Reference Materials for further reading to help you better understand the required activities.  Follow these activities on your own and answer the self-check at the end of each learning outcome.  Get the answer key from your instructor and check your work honestly.  If you have questions, please don’t hesitate to ask your facilitator for assistance.

Recognition of Prior Learning (RPL)

You may already have some or most of the knowledge and skills covered in this module because you have:

- been working for someone

- already completed training in this area

If you can demonstrate to your trainer that you are competent in a particular skill or skills, talk  to him/her about having them formally recognized so you won’t have to do the same training again.  If you have qualifications or Certificates of Competency from previous trainings, show them to your trainer.  If the skills you acquired are still relevant to this module, they may become part of the evidence you can present for RPL.

At the end of this learning material is a Learner’s Diary, use this diary to record important dates, jobs undertaken and other workplace events that will assist you in providing further details to your trainer or assessors.  A Record of Achievement is also provided for your trainer to complete once you completed the module.

This learning material was prepared to help you achieve the required competency, in Performing Periodic Maintenance of Suspension System NC I. This will be source of information for you to acquire the knowledge and skills in this particular trade independently and your own pace with minimum supervision or help from your instructor.

In doing the activities to complete the requirements of this module, please be guided by the following:

- Talk to your trainer and agree on how you will both organize the training under this module.  Read through the module carefully.  It is divided into sections that cover all the skills and knowledge you need to successfully complete.

- Work through all information and complete the activities in each section.  Read the information sheets and complete the self-checks provided in this module.

- Most probably your trainer will also be your supervisor or manager. He/She is there to support you and show you the correct way to do things.  Ask for help.

- Your trainer will tell you about the important things you need to consider when you are completing the activities and it is important that you listen and take notes.

- You will be given plenty of opportunities to ask questions and practice on the job.  Make sure you practice your new skills during regular work shifts.  This way you will improve both your speed and memory and also your confidence.

- Talk to more experienced work mates and ask for their guidance.

- Use self-check questions at the end of each section to test your own progress.

- When you are ready, ask your trainer to watch you perform the activities outlined in this module.

- As you work through the activities, ask for written feedback on your progress.  Your trainer keeps feedback/pre-assessment reports for this reason.  When you have completed this learning material and feel confident that you have had sufficient knowledge and skills, your trainer will arrange an appointment with a registered assessor to assess you.  The results of the assessment will be recorded in your Competency Achievement Record.

NATIONAL CERTIFICATE LEVEL 1

QUALIFICATION LEVEL

COMPETENCY-BASED LEARNING MATERIALS

| No. | Basic Competencies | Module Title | Code |
| --- | --- | --- | --- |
|  | Receive and Response to Workplace Communication | Receiving and Responding to Workplace Communication | 500311101 |
|  | Work with Others | Working with Others | 500311102 |
|  | Demonstrate Work Values | Demonstrating Work Values | 500311103 |
|  | Practice Housekeeping Procedures | Practicing Housekeeping Procedures | 500311104 |

| No. | Common Competencies | Module Title | Code |
| --- | --- | --- | --- |
|  | Validate Vehicle Specification | Applying Appropriate Sealant/Adhesive | ALT723201 |
|  | Move and Position Vehicle | Moving and Positioning Vehicle | ALT832212 |
|  | Utilize Automotive Tools | Performing Housekeeping | ALT723214 |
|  | Perform Mensuration and Calculation | Performing Mensuration and Calculation | ALT723215 |
|  | Utilize Workshop Facilities And Equipment | Performing Safety Practices | ALT723216 |
|  | Prepare Servicing Parts and Consumables | Reading, Interpreting and Applying Specifications and Manuals | ALT723217 |
|  | Prepare Vehicle For Servicing and Releasing | Using and applying Lubricant/Coolant | ALT723218 |

| No. | Core Competencies | Module Title | Code |
| --- | --- | --- | --- |
|  | Perform Pre-Delivery Inspection | Performing Pre-Delivery Inspection | ALT723372 |
|  | Perform Periodic Maintenance of Engine | Performing Periodic Maintenance of Engine | ALT723373 |
|  | Perform Periodic Maintenance of Drive Train | Performing Periodic Maintenance of Drive Train | ALT723374 |
|  | Perform Periodic Maintenance of Brake System | Performing Periodic Maintenance of Brake System | ALT723375 |
|  | Perform Periodic Maintenance of Suspension System | Performing Periodic Maintenance of Suspension System | ALT723376 |
|  | Perform Periodic Maintenance of Steering System | Performing Periodic Maintenance of Steering System | ALT723377 |

TABLE OF CONTENTS

HOW TO USE THIS COMPETENCY-BASED LEARNING MATERIAL………i

LIST OF COMPETENCIES…………………………………………………....…...ii

TABLE OF CONTENTS………………………………………………..…...……iii

MODULE CONTENTS………………………………………………………..…...v

LEARNING OUTCOME NO. 1

Perform periodic maintenance of suspension system.…....……...…1

LEARNING EXPEREINCES………………………………………….………….2

INFORMATION SHEET 5.1-1  Toola, Equipment and Materials…..5

Self-Check 5.1-1………………………………………………………......8

Answer Key 5.1-1………………….…………………………………...…….….9

Task sheet 5.1-1 ………………………………………………………………..

INFORMATION SHEET 5.2-1                                                                                   Inspection procedures based on repair manual…........................................10

Self-Check 5.2-1       ………………………………………………………..…...12

Answer Key 5.2-1   …………………………………………………………........13

INFORMATION SHEET 5.2-2  Proper torque specifications........................14

Self-Check 5.2-2………………………………………………………………..16

Answer Key 5.2-2 ………………………………………………………....….17

Task sheet 5.2-2 ………………………………………………………………..

INFORMATION SHEET 5.2-3  Proper torque specifications.......................18

Self-Check 5.2-3…………………………………………..21

Answer Key 5.2-3………………………………………………...…………….22

INFORMATION SHEET 5.3-1

Waste management fundamentals ………………………………….23

Self-Check 5.3-1…………………………………………………………..…25

Answer key 5.3-1…………………………………………………………….26

INFORMATION SHEET 5.3-2 Initial quality inspection ………….….27

Self-Check 5.3-2…………………………………………………..…………28

Answer key 5.3-2…………………………………………………………….29

INFORMATION SHEET  5.3- 3                                                              Accomplishment of job/repair order and 5s.………………………….......30

Self-Check 5.3-3……………………………………………………………..35

Answer Key 5.3-3………………………………………...……….…………36

QUALIFICATION		:	Automotive Servicing NC I

UNIT OF COMPETENCY	:	Perform Periodic Maintenance of Suspension

System

MODULE TITLE 		: 	Performing Periodic Suspension of Steering

System

PERFORMANCE  CRITERIA:

- LO 1:  Perform Pre-Periodic Maintenance of Suspension System

  - Job requirements are determined based on suspension system repair order.

  - Servicing information is sourced from service manual.

  - Tools, equipment and materials are prepared based on suspension system repair order.

1.4 Hazards and risks associated in the workplace are managed following OSHS.

1.5 Protective covers are installed based on standard operating procedure

- LO 2:  Apply Periodic Maintenance Procedure

2.1    Steering system components are inspected according to  manufacturer’s

service workshop manual.

2.2    Findings and recommendations are reported to the immediate superior

following company’s standard procedures.

2.3    Power steering fluid is replace based  on manufacturer’s service workshop

manual

  - Steering wheel free-play inspection is conducted based on manufacturer’s

service  workshop manual.

  - Defects and damages are reported to the to the immediate supervisor.

  - Safety practices are applied following OSHS.

.

- LO 3: Perform work to Completion

3.1    Wastes are disposed  according to good housekeeping practices

3.2    Initial quality inspection is performed based on workplace procedure

3.3    Job done is written down in the repair order.

3.4    Workplace is restored according to company’s standard procedure..

3.5    Safety practices are applied following  OSHS.

3.6    Tools and equipment are checked, cleaned and stored following 5s

QUALIFICATION		:	Automotive Servicing NC I

UNIT OF COMPETENCY	:	Perform Periodic Maintenance of Suspension

System

MODULE TITLE 		: 	Performing Periodic Maintenance of Suspension

System

LEARNING OUTCOME   1	: 	Perform Pre-Periodic Maintenance of 			                                           Suspension System

PERFORMANCE  CRITERIA

LO 1:  Perform Pre-Periodic Maintenance of Suspension System

  - Job requirements are determined based on suspension system repair order.

  - Servicing information is sourced from service manual.

  - Tools, equipment and materials are prepared based on suspension system repair order.

1.4 Hazards and risks associated in the workplace are managed following OSHS.

1.5 Protective covers are installed based on standard operating procedure

.

RESOURCES:

- Jack Erjavec and Robert scharff, Automotive Technology, A system Approach, 2nd Ed.

- Crouse, W.H. and Anglin, D.L., Automotive Mechanics, 10th Ed.,

- Santos, G.N. and Lebron G.B., Chemistry.

- Don Kowels, Basic Automotive Service and Maintenance.

QUALIFICATION		:	Automotive Servicing NC I

UNIT OF COMPETENCY	:	Perform Periodic Maintenance of Suspension

System

MODULE TITLE 		: 	Performing Periodic Maintenance of Suspension

System

LEARNING OUTCOME   1	: 	Perform Pre-Periodic Maintenance of 			                                           Suspension System

Contents:

- Tools, equipment and materials preparation  based on suspension system repair order

- Service/repair manual usage.

- Managing risk and hazards in the workplace

Assessment Criteria

- Job requirements are determined based on suspension system repair order.

- Servicing information is sourced from service manual.

- Tools, equipment and materials are prepared based on suspension system repair order.

- Hazards and risks associated in the workplace are managed following OSHS.

- Protective covers are installed based on standard operating procedure

Conditions

The following resources should be provided

1.Workplace: Real or simulated work area

2. Appropriate Tools & equipment

3. Materials relevant to the activity

4. Repair manuals and related reference materials

Assessment Method

- Demonstration with oral questioning

- Written exam

- Interview

INFORMATION SHEET		:	5.1-1

QUALIFICATION		:	Automotive Servicing NC I

UNIT OF COMPETENCY	:	Perform Periodic Maintenance of Steering System

MODULE TITLE 		: 	Performing Periodic Maintenance of Steering

System

LEARNING OUTCOME   1	: 	Perform Pre-Periodic Maintenance of Steering

System

Tools, equipment and materials

Learning Objectives:

After reading this INFORMATION SHEET, YOU MUST be able to:

- Prepare tools, equipment and materials based on repair order

- Familiarize with the usage of repair manual

- Manage/identify hazards and risk in the workplace.

Basics

Automotive repairs require the use of a variety of tools and measuring instruments. These tools are manufactured for use in a particular way, and accurate work and safety can only be assured if they are used correctly.

- Choose a tool according to the type of job

For removing and replacing bolt / nut or removing parts. 
It is usual to use a socket wrench set for automotive repairs. If a socket wrench set cannot be used due to work space restrictions, select a box end wrench or an open end wrench in that order.

Choose tools according to the speed with which the job can be done.

- A socket wrench is useful in that it can be used to turn bolt / nut without the need for repositioning. This allows for quick turning of bolt / nut.

- A socket wrench can be used in various ways depending on the type of handle that is fitted.

NOTICE:

| 1. | The ratchet handle is suitable for use in a confined space. However, due to the ratchet's construction, it is not possible to obtain particularly high torque. |
| --- | --- |
| 2. | The sliding handle requires a large work space but offers the fastest operating speed. |
| 3. | The spinner handle allows for quick work, with setting up the handle. However the handle is long and difficult to use in a confined space. |

Choose tools with consideration for the size of turning torque

- If a large torque is required for final tightening or initial loosening of a bolt / nut, use a wrench that allows a lot of strength to be applied.

NOTICE:

- The amount of strength that can be applied is dependent on the length of the shaft of the wrench. The longer the shaft, the larger the torque that can be gained from a small amount of force.

- If an excessively long shaft is used, there is a danger of over torque, and the

- bolt may be snapped.

Auto lift

Lifts the vehicle high so that the technician can assume a comfortable posture to work under the vehicle. There are three types of lifts with different lifting functions, support columns, and support methods.

Instructions

- Setting

|  | - Position the vehicle in the center of the lift. |
| --- | --- |
|  | - Fit the plates and arms to positions as indicated in the repair manual. |

NOTICE:

- Swing arm type

- Adjust the supports until the vehicle is horizontal.

- Always lock the arms.

- 4- column lift type

- Use wheel stoppers and safety mechanism.

- Plate type

- Use the plate lift attachment as indicated in the repair manual.

CAUTION:

- Align the position of the plate lift attachments with the areas of the vehicle to be supported.

- Do not allow the plate lift attachments to stick out of the plates.

Auto Lift

2. Lift up / Lift down

- Always perform a safety check before raising or lowering the lift, and signal to others that the lift is about to operate.

- Once the tires are a little off the ground, check that the vehicle is properly supported.

CAUTION:

- Take any luggage put of the vehicle, and raise it empty.

- Check that no parts are in the way, other than the supporting parts.

- Never lift any vehicle that exceeds the weight limit for the lift.

- Vehicles with air suspension need special handling due to their construction. Please refer to the repair manual for instructions.

- Do not move the vehicle while it is raised.

- Take care when performing removal and replacement of heavy parts as the center of gravity of the vehicle may change.

- Do not lift with doors open.

- If the operation is to be left unfinished for a period of time, always lower the vehicle.

Jack

Jack 
Uses hydraulic pressure to raise one end of the vehicle.

- Operating the handle increases the oil pressure and causes the arm to lift.

- Some models use air pressure to increase the oil pressure.

- There are various models available with different lifting capabilities (measured in tons.)

Rigid rack 
Support the vehicle that is raised by a jack.

- Height can be adjusted by changing the pin position.

|  | Release handle |  | Handle |  | Arm |
| --- | --- | --- | --- | --- | --- |
|  | Plate |  | Roller |  | Caster |
|  | Lifting button (air type) |  | Air hose (air type) |  | Pin |
|  | Positioning hole |  |  |  |  |

Instructions

1. Preparation

| (1) | Check the vehicle's jacking points and stand support points of rigid racks in the repair manual before jacking up. |
| --- | --- |
| (2) | Make sure rigid racks are set to the same height. Position them near the vehicle. |
| (3) | Place wheel stoppers in front of the front left and right tires (if the vehicle is to be lifted from behind.) |

2. Jacking up

| (1) | Securely tighten the release handle. |
| --- | --- |
| (2) | Place the garage jack in the specified position and lift the vehicle, paying attention to the direction it is facing. |

NOTICE:

- It is usual to jack up a vehicle from the rear end. However, the order can change depending on the model.

- Use a jack adapter for 4WD vehicles with offset differential gears.

CAUTION:

- Always work on a flat surface, and with all luggage removed from the vehicle.

- Always use a jack stand when jacking up. Do not enter the area under the vehicle until the rigid rack has been fitted.

- Do not use multiple garage jacks at one time.

- Do not lift any vehicle that exceeds the maximum permitted load for the jack.

- Vehicles with air suspension need special handling due to their construction. Please refer to the repair manual for instructions.

3. Support with rigid rack

| (1) | Position the legs of the rack as shown, and align the rubber grooves on the rigid rack to the body. |
| --- | --- |
| (2) | Re- check the height of the rack so that the vehicle is in a horizontal position. |
| (3) | Loosen the release handle slowly, and once the load has been placed on the rigid rack, strike the legs of the rack gently with a hammer to check that they are all touching the ground. |
| (4) | Remove the jack after checking. |

CAUTION:

- Do not enter the area under the vehicle while lifting or removing the rigid rack.

4. Jacking down

| (1) | Place the garage jack in the specified position, and raise the vehicle, paying attention to the direction it is facing. |
| --- | --- |
| (2) | Remove the rigid rack. |
| (3) | Loosen the release handle slowly, and lower the arm gently. |
| (4) | Once the tires have reached the ground completely, apply wheel stoppers. |

NOTICE:

- It is usual to jack a vehicle down from the front end. However, the order can change depending on the model.

CAUTION:

- Perform safety checks before raising or lowering a vehicle, and indicate to other people that the operation is about to happen. Check that nothing is below the vehicle before lowering it.

- Loosen the release handle slowly, and lower the arm gently.

- When the garage jack is not being used, lower the arm and stand the handle up.

Service Manuals

As automobiles evolve, new systems and features are incorporated in the new models. Thus, it is becoming increasingly difficult for technicians to service the highly complex vehicles by relying on their experience alone.

To inform the service personnel around the world of the proper service procedures and new technologies, Car Manufacturer publishes various types of manuals.

|  | Repair manuals |
| --- | --- |
|  | EWD (Electrical Wiring Diagram) |
|  | SST (Special Service Tools) List |
|  | NCF (New Car Features) |
|  | SDS (Service Data Sheets) |
|  | Owner's manuals |
|  | Others |

Toyota Parts Numbers

To correctly identify the components of all vehicles, a part number that consists of 10 or 12 digits of alphanumeric characters is assigned to every part. 
The details of the part numbers are given in the

PARTS CATALOG GUIDE*. 
* Published by Toyota's service parts administration division.

Basic part number 
Design number 
(-00)

Parts Catalogs

Although part numbers are assigned to all parts, they are not necessarily indicated on the parts themselves. All part numbers can be looked up in the parts catalogs. 
The parts catalogs come in the following three types. Refer to PARTS CATALOG GUIDE* for the proper method for using the catalogs. 
* Published by Toyota's service parts administration division.

|  | Microfiche Parts Catalog |
| --- | --- |
|  | Paper Parts Catalog |
|  | Electronic Parts Catalog (CD-ROM) |

Searching Parts Numbers

Information used for searching Parts Catalogs
To look up a telephone number in a telephone directory, you will need to know beforehand the name of the person or the address for the number you are seeking.  
Similarly, to look up a part number in a Parts Catalog, you will need to have some pieces of information about the vehicle.

For example:

- Model code

- Body color code

- Trim code

- Transmission code

- Axle code

These pieces of vehicle information are indicated on the nameplate or the certification regulation label of the vehicle.

Name plate 
The nameplate is also known as a "manufacturer plate". Its contents vary according to the destination of the vehicle.

The nameplate of a passenger car is located on the cowl panel. The repair manual shows the exact location of the nameplate.

Numbers indicated on the vehicle 
Frame number or VIN (Vehicle Identification Number) is punched on the engine compartment or chassis, etc. in addition to the nameplate. Refer to the repair manuals because the punched locations vary with the vehicle types.

VIN (Vehicle Identification Number) 
The VIN, which is required by the laws of the country to which the vehicle is exported, is punched on the nameplate or vehicle body. 
A VIN contains 17 digits consisting of WMI, VDS, and VIS.

VIS is the same as the serial number of a frame number, which includes the time of production.

The VIN is used in the following countries:

- The USA and Canada (including U.S. trust territories)

- Europe (including French trust territories)

- Australia, NZ, Brazil, Hong Kong, Indonesia, South Africa, etc.

The diagram on the left indicates a VIN for the USA and Canada.

Vehicle Specification

The vehicle specifications, which consist of vehicle size, performance, and other pieces of information, are included in the product brochure and NCF (New Car Features) manuals, etc.
Service technicians should be acquainted with all the items of the vehicle specifications.

SELFCHECK 			:	5.1-1

QUALIFICATION		:	Automotive Servicing NC I

UNIT OF COMPETENCY	:	Perform Periodic Maintenance of Suspension

System

MODULE TITLE 		: 	Performing Periodic Maintenance of Suspension

System

LEARNING OUTCOME   1	: 	Perform Pre-Periodic Maintenance of 			                                           Suspension System

I. TRUE/FALSE: Read each statement below carefully. Place a T on the line if you think a statement it TRUE. Place an F on the line if you think the statement is FALSE. If you have questions, raise your hand and ask your teacher.

___________ 1. Remove and re-install bolts and nuts by using the tool according to the part. Select the tool by the following order of priority.
(1)socket wrench set --> (2)boxend wrench --> (3)openend wrench.

___________ 2. When using a rigid rack, support the vehicle at its front/rear end of rocker panel.

___________ 3. During the final tightening of a bolt or nut, use a torque wrench to tighten it to the specified torque in accordance with the repair manual.

___________ 4. There is a method for raising a vehicle by simultaneously using two jacks.

___________ 5. When measuring the dimensions of a part, use a measuring instrument with a measuring precision that is suited to the required precision of the part.

II. Identification: Match the tools shown below with the corresponding names in the word group. Write the letter on the space provided.

  - 2.

  - 4.

ANSWER SHEET 		:	5.1-1

QUALIFICATION		:	Automotive Servicing NC I

UNIT OF COMPETENCY	:	Perform Periodic Maintenance of Suspension

System

MODULE TITLE 		: 	Performing Periodic Maintenance of Suspension

System

LEARNING OUTCOME   1	: 	Perform Pre-Periodic Maintenance of 			                                           Suspension System

I. TRUE/FALSE: Read each statement below carefully. Place a T on the line if you think a statement it TRUE. Place an F on the line if you think the statement is FALSE. If you have questions, raise your hand and ask your teacher.

TRUE   1. Remove and re-install bolts and nuts by using the tool according to the part. Select the tool by the following order of priority.
(1)socket wrench set --> (2)boxend wrench --> (3)openend wrench.

TRUE 2. When using a rigid rack, support the vehicle at its front/rear end of rocker panel.

TRUE 3. During the final tightening of a bolt or nut, use a torque wrench to tighten it to the specified torque in accordance with the repair manual.

FALSE 4. There is a method for raising a vehicle by simultaneously using two jacks.

TRUE 5. When measuring the dimensions of a part, use a measuring instrument with a measuring precision that is suited to the required precision of the part.

II. Identification: Match the tools shown below with the corresponding names in the word group. Write the letter on the space provided.

1.H		2.B

3.A	 	4.C

TASK SHEET 		:	5.1-1

QUALIFICATION		:	Automotive Servicing NC I

UNIT OF COMPETENCY	:	Perform Periodic Maintenance of Suspension

System

MODULE TITLE 		: 	Performing Periodic Maintenance of Suspension

System

LEARNING OUTCOME   1	: 	Perform Pre-Periodic Maintenance of 			                                           Suspension System

Title	: Standard Lifting Operation

Performance Objective:

Given the materials/equipment needed for practice. You should be able to perform standard lifting position and operation.

Supplies/ Materials:

- CBLM on Servicing Suspension system

- Information sheets

Equipment	:

- Two post lifter

- Plate Type lifter

Procedure:

- Make sure the swing arms are moved backwards and out of the way. Move the vehicle into the service bay and park it in between the two posts that make up the lift.

- Locate the vehicle's lift points underneath. Swing the arms under the vehicle and position the pads under the lift points. Consult the owner's manual for your vehicle if you do not know where the lift points are. Check the screw pads to insure the swing arms make contact with the lift points at the same time.

- Locate and press the button on the power unit to raise the vehicle until the tires are 6 inches above the ground. Move to the rear of the vehicle and push the rear bumper up and down. If the vehicle bounces on the pads or otherwise feels unstable, lower the vehicle to the ground and reposition the pads as necessary

- Press the button on the power unit once more to raise the vehicle to the desired height. Lower the lift until it rests on the safety lock.

Lower the vehicle from the lift by raising it a half-inch to free it from the safety lock. Pull the cable release and hold on to the lowering handle. The vehicle will slowly descend to the ground.

Safety Pre-Caution

- Never work under a lift while it is in motion or if the safety locks are not engaged or inoperable. Doing so can lead to serious injury or death.

Assessment Method:

Direct Observation

Performance Criteria Checklist 1.1-1

| CRITERIA D Did you…. | YES | NO |
| --- | --- | --- |
| - Prepare the supplies, materials, tools and equipment needed? |  |  |
| - Correctly installed the lifter arms on the jacking points? |  |  |
| - Check the stability of the vehicle before lifting? |  |  |

QUALIFICATION		:	Automotive Servicing NC I

UNIT OF COMPETENCY	:	Perform Periodic Maintenance of Steering System

MODULE TITLE 		: 	Performing Periodic Maintenance of Steering

System

LEARNING OUTCOME   2	: 	Apply Periodic Maintenance Procedure

PERFORMANCE CRITERIA

LO 2:  Apply Periodic Maintenance Procedure

2.1    Steering system components are inspected according to  manufacturer’s

service workshop manual.

2.2    Findings and recommendations are reported to the immediate superior

following company’s standard procedures.

2.3    Power steering fluid is replace based  on manufacturer’s service workshop

manual

  - Steering wheel free-play inspection is conducted based on manufacturer’s

service  workshop manual.

  - Defects and damages are reported to the to the immediate supervisor.

  - Safety practices are applied following OSHS.

RESOURCES:

Jack Erjavec & Robert scharff, Automotive Technology, A system Approach, 2nd Ed.

Crouse, W.H. and Anglin, D.L., Automotive Mechanics, 10th Ed.,

Santos, G.N. and Lebron G.B., Chemistry.

Don Kowels, Basic Automotive Service and Maintenance.

INFORMATION SHEET 			: 	5.2-1

QUALIFICATION		:	Automotive Servicing NC I

UNIT OF COMPETENCY	:	Perform Periodic Maintenance of Steering System

MODULE TITLE 		: 	Performing Periodic Maintenance of Steering

System

LEARNING OUTCOME   2	: 	Apply Periodic Maintenance Procedure

Suspension system components and operation

Learning Objectives:

After reading this INFORMATION SHEET, YOU MUST be able to:

- Understand suspension system components and operation

- Apply inspection procedures based on repair manual

- Appreciate the importance of torque specification

Description

The suspension connects the body of the vehicle with the wheels, and carries out the following functions:

The suspension system joins the wheels to the body or the frame in order to support the vehicle physically.

- Improves the ride by dampening the impact that the tires receive from the road surface.

- Ensures driving stability.

- Springs – neutralize the shock from the road surface

- Shock absorber (damper) - Acts to improve riding comfort by limiting the free oscillation of the springs.

- Stabilizer (sway bar or anti roll bar) - prevents the lateral swaying of the vehicle.

- Linkage - Acts to hold the above components in place and to control the longitudinal and lateral movements of the wheels.

Types of springs
The role of the springs is to absorb the impact from the road surface and reduce the vibration that transmits to the body.

Shock Absorbers

The shock absorbers restrain the movement of springs by the resistance of the oil that flows through a path in the piston. They also absorb the vibrations of the vehicle body, and give a good ride.

Ball Joint

Ball joints bear vertical as well as horizontal loads, and also act as pivots for the steering knuckles when the steering wheel is turned.

Stabilizer Bar

When the vehicle turns a corner, it leans to the outside because of the centrifugal force. The stabilizer bar controls it with the power of the spring that twists, and keeps the tires close to the ground. It also operates if the tires on one side runs onto a different on level.

When the vehicle leans and the tires sink to one side, the stabilizer bar is twisted and operates as a spring, which lifts up the tires) the body on the sinking side. 
In case the tires on both sides sink equally, the stabilizer bar does not operate as a spring because it is not twisted.

TYPES OF SUSPENSION

There are two types of suspensions, depending on how they support the wheels.

- Rigid axle suspension

Both wheels are joined to a single axle, which is mounted to the body via springs.
Because both wheels and the axle move vertically in unison, the movements of the wheels affect each other. This type of suspension has a simple and solid construction.

- Independent suspension

Each wheel is supported by an independent arm, which is mounted to the vehicle body via a spring. This type of suspension can efficiently absorb the roughness of the road, and gives an excellent ride because each wheel moves up and down independently from the other wheel.

Types of rigid axle suspension

- Torsion beam type

Consists of left and right trailing arms connected to the crossbeam. Similar to the link-type suspension, the springs support only the vertical force. It has a simple construction and gives a good ride. This type of suspension is used on the rear suspension of lightweight FF vehicles.

- 4 link type

Upper and lower control arms are mounted longitudinally to the vehicle body at each end of the axle, and another arm is mounted latitudinally from one end of the axle to the body. These arms support the longitudinal and latitudinal forces that act on the axle, enabling the springs to support only the vertical force. The construction of this type of suspension is slightly complex, although it gives a better ride than the leaf springs. It is used on the rear suspension of 1-box, SUV (Sports Utillity Vehicle), FR, and 4WD vehicles.

- Leaf spring type

Each end of the axle that joins both wheels is attached to a leaf spring. The leaf springs, which are parallel to each other, are mounted longitudinally to the vehicle body. The force that acts on the axle is transmitted to the body via the springs. This type is used primarily on the rear suspension of vans and trucks due to its simple and sturdy construction.

Types of independent suspension

- Macpherson strut type

This is a type of suspension that does not have an upper arm, resulting in a simpler construction than the double wishbone type.
It can be serviced more easily because it has fewer components.
It is used primarily for the front suspension of FF vehicles.

- Double wishbone type

Consists of upper and lower arms that support the wheel and a knuckle that links the arms. The arms receive the longitudinal and latitudinal forces, enabling the spring to support only the vertical load. 
Although this construction is complex because it involves many parts, it is highly rigid to firmly support the wheels.
Because the layout of this suspension can be designed freely, it offers both a comfortable ride and driving stability. It is widely used for FR vehicles.

- Semi trailing arm type

The rear suspension arm is mounted at a prescribed angle to the rear suspension member in order to withstand a greater lateral force. This design offers the same effect as if the arm had been made more rigid. It is used on the rear suspension of some FR vehicles.

Tires

Among the numerous parts that are used on an automobile, tires are the only ones that come in contact with the road surface, and support three basic efficiencies: driving, turning and stopping.
Maintenance items include tire inspection (exterior damage, tread depth, and wear conditions), adjustment of air pressure, and the rotation of the tires.

Types and Features of Tires

Specification Coding System

The size, performance, and the construction of the tire are indicated on the sidewall of a tire. 
The diagram on the left shows the names of the sizes of the various areas of a tire.

Aspect ratio 
The ratio of the height of the cross section of the tire to the width (which is rendered 100) is indicated as a percentage (%).

- A tire with a high aspect ratio 
The cornering performance is slightly poor. 
It gives a better ride, and is suited to family cars.

- A tire with a low aspect ratio 
It gives a slightly poorer ride. It is better suited to sports cars because the cornering performance is better.

Disc Wheels

A disc wheel is a disc-shaped part to which a tire is mounted. With the tire, it supports three basic functions: driving, turning and stopping.

Pressed-steel disc wheel 
This wheel is made of pressed steel. It is heavy, but strong.

Cast light-alloy disc wheel 
This wheel is made of aluminum. It is lightweight and offers excellent design performance. Compared to the steel wheel, has lower shock resistance

Disc wheels specification coding

The size of the wheel is indicated on the rim of the disc wheel.

How to read the wheel size

*1: Codes "J" and "JJ" are frequently used, depending on the shape of the wheel rim flange. The JJ is slightly higher than the J, which makes the tire less susceptible to coming loose.

SELFCHECK 			:	5.2-1

QUALIFICATION		:	Automotive Servicing NC I

UNIT OF COMPETENCY	:	Perform Periodic Maintenance of Suspension

System

MODULE TITLE 		: 	Performing Periodic Maintenance of Suspension

System

LEARNING OUTCOME   2	: 	Apply Periodic Maintenance Procedure

- Identification: Identify the following. Write your answer on the

space provided.

- Principal components of suspension system

- _______________________

- _______________________

- _______________________

- _______________________

- 2 types of suspension depending on how they support the wheels

- __________________________________________

- __________________________________________

- Identify the following:

A.  ___________________________________________

B.  ___________________________________________

C.  ___________________________________________

ANSWER KEY 		:	5.2-1

QUALIFICATION		:	Automotive Servicing NC I

UNIT OF COMPETENCY	:	Perform Periodic Maintenance of Steering System

MODULE TITLE 		: 	Performing Periodic Maintenance of Steering

System

LEARNING OUTCOME   2	: 	Apply Periodic Maintenance Procedure

- Identification

- Principal components of suspension system

- Spring

- Shock absorber

- Stabilizer

- Linkage

- 2 types of suspension depending on how they support the wheels

1. Rigid axle suspension

2. Independent suspension

C. Identify the following:

A. Torsion bean type

B. 4- link type

C. Leaf spring type

- Macpherson strut

- Double wishbone type

- Semi- trailing arm type

INFORMATION SHEET 		:	5.2-2

QUALIFICATION		:	Automotive Servicing NC I

UNIT OF COMPETENCY	:	Perform Periodic Maintenance of Steering

System

MODULE TITLE 		: 	Performing Periodic Maintenance of

Steering

System

LEARNING OUTCOME   2	: 	Apply Periodic Maintenance Procedure

Inspection procedures

Learning Objectives:

After reading this INFORMATION SHEET, YOU MUST be able to:

- Perform inspection procedures based on repair manual

- Identify faults on suspension system

- Appreciate the importance of various inspections.

Checking the suspension

Besides supporting the body, the suspension also controls the movement of the wheels in relation to the body. The suspension is composed of various types of components each with their own purposes and functions. Therefore in order for the suspension to operate effectively, it is necessary that all of its parts be installed correctly, and that there be no looseness, play, bending, weakening, wear, etc. Particularly, bending of suspension arms or wear of bushings and ball joint causes a number of problems with wheel alignment.

Suspension springs

- Check for cracks, damage, or wear in the springs.

- Check for weakening springs

One method for checking for weakening spring is by checking the vehicle height. Take the car to a level place and inflate all the tires to prescribed pressure. Next, measure the height of the vehicle at the points specified in the repair manual. If the height is lower than the prescribed value, the springs may be weakened by age.

- Check the installation of the springs

- Coil springs – the insulator should be installed properly and the spring should be firmly against the stopper.

- Leaf Spring

- Leaves should be checked for displacement in the lateral and longitudinal directions. Also, the spring clips should not be deformed.

- Check the U bolts and clips for looseness or damage

- Check the spring mounts for play in the longitudinal and lateral directions by prying carefully with a lever.

SUSPENSIONS LINKAGES

1. Pry the mounts of the body or frame side with a lever, checking for play in the longitudinal and lateral directions.

Front Suspension

- Upper Suspension Arms

- Lower Suspension Arms

- Strut Bars

- Stabilizer Bar

Rear Suspension

- Upper Control Arms

- Lower Control Arms

- Suspension Arms

- Lateral Control Arms

- Stabilizer Bar

If there is play, there could be improper tightening of the arm or rod, or the bushings could be worn

2. In the case of the strut type suspension, check for play in the upper support by pushing and pulling on the tires with both hands placed on top of the tire as shown here.

Check for damage to the installation area of the front suspension upper support.

3. Check the suspension arms for deformation.

4. Check the tightness of each arm and rod. Each arm and rod should be tightened to the prescribed tightening torque. Insufficient torque could throw the suspension geometry out of alignment, while excessive torque could cause deformation of the bushings and accelerate bushing aging and wear.

- Check the rivets and bolts of the spring brackets and suspension installation brackets for looseness. Also, make sure that the brackets are not cracked or damaged.

- Check ball joints for play.

SHOCK ABSORBERS

- Check for fluid leakage, deformation and damage.

- Check for cracks, damage or play in the installation areas.

- Check the operation of the shock absorbers. Move each corner of the vehicle up and down. The up-and-down motion should stop immediately after the hands released.

WHEEL ALIGNMENT

In order to confirm that the wheels are being held in the proper position and at the proper angle by the suspension, check the front wheel alignment and rear wheel tracking. If the alignment is improper, be sure to adjust so the values are within the prescribed range. If the adjustment cannot be made, check the suspension, steering system and axle.

- Front Wheel Alignment

- Rear Wheel Tracking

The left and right wheels should be installed so that they are symmetrical with respect to the vehicle’s centerline.

TROUBLESHOOTING

The purposes of the suspension are to control irregular vibrations, shocks and oscillations transmitted to the body or the wheels while the car is in motion in order to provide good riding comfort and driving stability. However, what must be stressed here is that the suspension is not the only factor influencing a car’s riding comfort or its driving stability: the tires and wheels also greatly influence riding comfort, while the tires and wheels, the steering system, the axles, etc., have great influence on driving stability. Another very important factor in obtaining satisfactory handling and driving stability is wheel alignment.

POOR RIDING COMFORT (RIDE TOO SOFT OR HARSH)

When we speak of poor riding comfort, we generally are referring to the following conditions:

- The car continuously and/or excessively pitches, rolls, bounces, etc.

- Shocks are transmitted from the road surface, giving a rough or harsh ride.

- The car tramps or hops excessively when it travels over bumpy, rutted roads.

- The car sways, giving a “spongy” ride.

NOTES

- The optimum size and play or tire is specified for each vehicle model. Tires of a large size or play are in themselves heavy. Therefore their use will increase the unsprung weight and causes riding comfort to suffer. At the same time, since this will increase tire rigidity, shocks form the road surface will not be absorbed well by the tires and a hard ride will result.

- If the tire inflation pressure is higher than that specified the tires rigidity will increase, resulting in a hard ride. On the other hand, if the inflation pressure is too low, the ride will be too soft.

- Check the following items and if there are any defects, replace the shock absorber.

- Check to make sure that the shock absorbers of the correct specification for the car are being used.

- Check the operation of the shock absorbers. If a shock absorber’s damping force is insufficient, it will not be able to absorb the oscillation of the suspension springs, and the body will bob around. Particularly when driving on rough roads, the riding comfort will be extremely poor.

- Check to make sure the shock absorbers are installed without any play or looseness. At the same time, check for wear or cracks in the bushings.

- If a shock absorber is not installed correctly, its sliding resistance will become excessively large, it will not function properly and the necessary damping force will not be obtained.

- Check to make sure there is no fluid leakage.

- if the springs constant or installation of the springs is not correct, the car will sway badly on rutted, bumpy roads and riding comfort will suffer.

- Check to make sure that the springs with the correct specification of the car are installed.

- Check to make sure there is no weakening or damaged to the springs.

Check to make sure that the installation of the springs is correct, and that there is no wear, weakening or cracks in the bushings, etc.

SELF-CHECK 		:	5.2-2

QUALIFICATION		:	Automotive Servicing NC I

UNIT OF COMPETENCY	:	Perform Periodic Maintenance of

Suspension   System

MODULE TITLE 		: 	Performing Periodic Maintenance of

Suspension System

LEARNING OUTCOME   2	: 	Apply Periodic Maintenance Procedure

- Identification: Identify the following. Write your answer on the

space provided.

- Maintenance items

Give atleast 3 inspection items for suspension springs.

1.

2.

3.

Give atleast 3 purpose of suspension

1. ___________________________________________________________

2. ___________________________________________________________

3. ____________________________________________________________

Fill the blanks. Based on the procedure of troubleshooting fill up the following.

ANSWER KEY  		:	5.2-2

QUALIFICATION		:	Automotive Servicing NC I

UNIT OF COMPETENCY	:	Perform Periodic Maintenance of

Suspension System

MODULE TITLE 		: 	Performing Periodic Maintenance of

Suspension System

LEARNING OUTCOME   2	: 	Apply Periodic Maintenance Procedure

AIdentification: Identify the following. Write your answer on the

space provided.

Maintenance items

Give atleast 3 inspection items for suspension springs.

1. Cracks

2. Wear in the springs

3. Damaged

Give atleast 3 purpose of suspension

1. To control irregular vibrations

2. To minimize shock from the road surface

3. To control oscillation transmitted to the body of the wheels while the car is in motion.

Fill the blanks. Based on the procedure of troubleshooting fill up the following.

QUALIFICATION		:	Automotive Servicing NC I

UNIT OF COMPETENCY	:	Perform Periodic Maintenance of Suspension

System

MODULE TITLE 		: 	Performing Periodic Maintenance of Steering

System

LEARNING OUTCOME   3	: 	Perform work to completion

PERFORMANCE  CRITERIA

LO 3: Perform work to Completion

3.1    Wastes are disposed  according to good housekeeping practices

3.2    Initial quality inspection is performed based on workplace procedure

3.3    Job done is written down in the repair order.

3.4    Workplace is restored according to company’s standard procedure..

3.5    Safety practices are applied following  OSHS.

3.6    Tools and equipment are checked, cleaned and stored following 5s

RESOURCES:

Jack Erjavec and Robert scharff, Automotive Technology, A system Approach, 2nd Ed.

Crouse, W.H. and Anglin, D.L., Automotive Mechanics, 10th Ed.,

Santos, G.N. and Lebron G.B., Chemistry.

Don Kowels, Basic Automotive Service and Maintenance.

QUALIFICATION		:	Automotive Servicing NC I

UNIT OF COMPETENCY	:	Perform Periodic Maintenance of Suspension

System

MODULE TITLE 		: 	Performing Periodic Maintenance of Suspension

System

LEARNING OUTCOME   3	: 	Perform work to completion

Contents:

- Waste management fundamentals

- Initial quality inspection

- Accomplishment of job/repair order and 5s.

Assessment Criteria

- Practicing good housekeeping

- Following standard

- Reporting results of inspection

- Performing initial quality inspection

- Accomplishing job/repair order

- Applying OSHS

Conditions

The following resources should be provided:

      - Workplace: Real or simulated work area

      - Appropriate Tools & equipment

      - Materials relevant to the activity

      - Repair manuals and related reference materials

Assessment Method

      - Demonstration with oral questioning

      - Written exam

      - Interview

Learning Outcome No. 3: Perform work to completion

| Learning Activities | Special Instructions |
| --- | --- |
| - Read Information sheet 5.3-1 on Waste management fundamentals | - You have read and understand the previous LO’s. You may undergo a test to check your learning. - Read and understand the INFORMATION SHEETS related to the task that will perform. - TASK SHEETS will help you to practice your skills. - The PERFORMANCE CRITERIA will guide and help you evaluate your work as you are practicing your skill. - Evaluate your own work using the PERFORMANCE CRITERIA. - When you are ready, present your work to your trainer to final evaluate and recording. - Strict follow the procedure to avoid untoward incidents. - Observe safety while performing the task. - Always return the tools/equipment after the work. - Apply 5’S. After performing the activities of LO2 you may proceed to LO3. |
| - Answer self-check 5.3-1 Compare your answer with Answer key 5.3-1 |  |
| - Read Information sheet 5.3-2 on Initial quality inspection |  |
| - Answer self-check 5.3-2 Compare your answer with Answer key 5. 3-2 |  |
| - Read Information sheet 5.3-3 on Accomplishment of job/repair order and 5s. |  |
| - Answer self-check 5.3-3 Compare your answer with Answer key 5.3-3 |  |

INFORMATION SHEET		         :      5.3-1

QUALIFICATION		:	Automotive Servicing NC I

UNIT OF COMPETENCY	:	Perform Periodic Maintenance of Suspension

System

MODULE TITLE 		: 	Performing Periodic Maintenance of Suspension

System

LEARNING OUTCOME 3  	: 	Perform work to completion Information

Waste management fundamentals

Learning Objectives:

After reading this INFORMATION SHEET, YOU MUST be able to:

- Understand the hazardous and non-hazardous waste of suspension.

- Apply proper disposal of hazardous and non-hazardous waste.

- Appreciate the importance of waste management

Description

Classification of wastes according to their origin and type.

- Municipal Solid wastes: Solid wastes that include household garbage, rubbish, construction & demolition debris, sanitation residues, packaging materials, trade refuges etc. are managed by any municipality.

- Bio-medical wastes: Solid or liquid wastes including containers, intermediate or end products generated during diagnosis, treatment & research activities of medical sciences.

- Industrial wastes: Liquid and solid wastes that are generated by manufacturing & processing units of various industries like chemical, petroleum, coal, metal gas, sanitary & paper etc.

- Agricultural wastes: Wastes generated from farming activities.  These substances are mostly biodegradable.

- Fishery wastes: Wastes generated due to fishery activities. These are extensively found in coastal & estuarine areas.

- Radioactive wastes: Waste containing radioactive materials. Usually these are byproducts of nuclear processes. Sometimes industries that are not directly involved in nuclear activities, may also produce some radioactive wastes, e.g. radio-isotopes, chemical sludge etc.

- E-wastes: Electronic wastes generated from any modern establishments. They may be described as discarded electrical or electronic devices. Some electronic scrap components, such as CRTs, may contain contaminants such as Pb, Cd, Be or brominated flame retardants.

Classification of Wastes according to their Effects on Human Health and the Environment.

Hazardous waste

- Substances unsafe to use commercially, industrially, agriculturally, or economically and have any of the following properties- ignitability, corrosively, reactivity & toxicity.

Non- hazardous waste

- Substances safe to use commercially, industrially, agriculturally, or economically and do not have any of those properties mentioned above. These substances usually create disposal problems.

In suspension system there are components that are classified as hazardous and non-hazardous waste like:

Hazardous waste                                         Non- hazardous waste

Front shock absorber                                  Front lower suspension arm

Rear shock absorber                                   Front stabilizer bar

Coil spring                                                  Axle beam

Front shock absorber

A representative type of mono-tube shock absorber is the DuCarbon type shock absorber, which is charged with high-pressure nitrogen gas (2.0-2.9 MPa; 20-30 kgf/ cm 2 ; 284-427 psi)

Characteristics of the DuCarbon type shock absorber

|  | Good heat radiation because the single tube is exposed directly to the atmosphere. |
| --- | --- |
|  | One end of the tube is charged with high-pressure gas, which is completely sealed off from the fluid by a free piston. This ensures that cavitation and aeration do not occur during operation, thereby providing more stable damping. |
|  | Operation noise is greatly reduced. |

Nitrogen gas is very harmful to our environment if inhalation of a Nitrogen enriched atmosphere (ie: loss of oxygen) may cause dizziness, drowsiness, nausea, vomiting, excess salivation, diminished mental alertness, loss of consciousness, and ultimately: death. That’s why it is considered as hazardous waste.

Coil Spring

Coil springs are made from rods of special spring steel, carbon and alloy wire formed into the shape of a coil. When a load is placed on a coil spring, the entire rod is twisted as the spring contracts. In this way, the energy of the external force is stored, and shock is cushioned.

Characteristics:

|  | The energy absorption rate per unit of weight is greater in comparison with leaf springs. |
| --- | --- |
|  | Soft springs can be made. |
|  | Since there is no inter-leaf friction as with leaf springs, there is no control of oscillation by the spring itself, so it is necessary to use shock absorbers along with them. |
|  | Since there is no resistance to lateral forces, linkage mechanisms to support the axle (suspension arm, lateral control rod, etc.) are necessary |

Elemental carbon is of very low toxicity. Health hazard data presented here is based on exposures to carbon black, not elemental carbon. Chronic inhalation exposure to carbon black may result in temporary or permanent damage to lungs and heart.

Servicing precautions

|  | Handling of shock absorbers |
| --- | --- |
| Since the oil seal, piston rod, and other components of the shock absorbers are made with extreme precision, the following precautions must be taken when handling them:. Exposed portions of the piston rod must not be scratched to prevent leakage of shock absorber fluid. In addition, the piston rod should be kept free of paint and oil.. In order to prevent damage to the oil seal caused by interference with the piston valve, the rod and cylinder should not be rotated with the shock absorber extended completely. Particular caution is required in the case of gas-filled shock absorbers since the piston rod is constantly pushed up by the pressure of the gas.. |

Handling of gas-filled shock absorbers

Since pressure is constantly applied on the inside of gas-filled shock absorbers, the following precautions should be taken in addition to those described above:. Do not attempt to disassemble non-disassemblable shock absorbers (including all DuCarbon type shock absorbers as well as low-pressure gas-filled shock absorbers in which the ring nut is caulked).. When disposing of gas-filled shock absorbers, first evacuate the gas..

Evacuation of gas

(1); DuCarbon type Drill a 2 to 3 mm hole about 10 mm from the bottom of the removed shock absorber cylinder before discarding in order to release the pressurized gas. (This gas is harmless, colorless, and odorless, but metal chips may fly out during drilling, so work carefully. A good safety practice is to put a vinyl bag around the end to be drilled, fastening it in place with a strong rubber band.). (2); Non-disassemblable MacPherson strut type In this type, in which the ring nut cannot be removed, lay the shock absorber assembly down horizontally and drill a 2 to 3 mm hole in the top of the absorber shell..

| 3) | Disassemblable MacPherson strut type <1>; Clamp the shock absorber in a vise.. <2>; Slowly loosen the ring nut three or four turns until the gas begins to leak out. If the gas is allowed to escape too quickly, the absorber fluid may come out as well.. <3>; Make sure that there is no gas remaining in the absorber before discarding. This can be determined by lifting the piston rod to the top of the cylinder and releasing it. If the piston falls back into the cylinder of its own weight, all of the gas has been exhausted.. |
| --- | --- |

|  | Installation of cartridge type low-pressure gas-filled shock absorbers |
| --- | --- |
| Although the shock absorber is replaced only as an assembly in the case of many low-pressure gas-filled MacPherson strut shock absorbers, such assembly replacement is not necessary in some models. Instead, the piston rod with cylinder can be removed and a replacement cartridge installed. The following precautions must be taken in such cases:. Since the piston rod with cylinder and the cartridge have different shapes, cartridges should be used that come with their own ring nut.. When disposing of the cartridge, first evacuate the gas following the same procedure as in the case of the non-disassemblable MacPherson strut type.. |

SELFCHECK	  	            :      5.3-1

QUALIFICATION		:	Automotive Servicing NC I

UNIT OF COMPETENCY	:	Perform Periodic Maintenance of Suspension

System

MODULE TITLE 		: 	Performing Periodic Maintenance of Suspension

System

LEARNING OUTCOME 3  	: 	Perform work to completion Information

Identification: Identify the following. Write your answer on the

space provided.

What are the 2 kinds of waste according to their effects on human health and environment?

- _________________________

- _________________________

Give the classification of waste according to their origin and type.

- _________________________

- _________________________

- _________________________

- _________________________

- _________________________

- _________________________

- _________________________

- These are made from rods of special spring steel, carbon and alloy wire, its main function is to support the axle.

ANSWER KEY	  	           :      5.3-1

QUALIFICATION		:	Automotive Servicing NC I

UNIT OF COMPETENCY	:	Perform Periodic Maintenance of Suspension

System

MODULE TITLE 		: 	Performing Periodic Maintenance of Suspension

System

LEARNING OUTCOME 3  	: 	Perform work to completion Information

- Identification: Identify the following. Write your answer on the

space provided.

What are the 2 kinds of waste according to their effects on human health and environment?

- Hazardous waste

- Non-hazardous waste

Give the classification of waste according to their origin and type.

- Municipal Solid wastes

- Bio-medical wastes

- Industrial wastes

- Agricultural wastes

- Fishery wastes

- Radioactive wastes

- E-wastes

- These are made from rods of special spring steel, carbon and alloy wire, its main function is to support the axle.

- Coil Springs

INFORMATION SHEET		         :      5.3-2

QUALIFICATION		:	Automotive Servicing NC I

UNIT OF COMPETENCY	:	Perform Periodic Maintenance of Suspension

System

MODULE TITLE 		: 	Performing Periodic Maintenance of Suspension

System

LEARNING OUTCOME 3  	: 	Perform work to completion Information

Initial Quality Inspection

Learning Objectives:

After reading this INFORMATION SHEET, YOU MUST be able to:

- Perform inspection procedures based on repair manual

- Identify faults on suspension system

- Appreciate the importance of various inspections.

Checking the suspension

Besides supporting the body, the suspension also controls the movement of the wheels in relation to the body. The suspension is composed of various types of components each with their own purposes and functions. Therefore in order for the suspension to operate effectively, it is necessary that all of its parts be installed correctly, and that there be no looseness, play, bending, weakening, wear, etc. Particularly, bending of suspension arms or wear of bushings and ball joint causes a number of problems with wheel alignment.

Suspension springs

- Check for cracks, damage, or wear in the springs.

- Check for weakening springs

One method for checking for weakening spring is by checking the vehicle height. Take the car to a level place and inflate all the tires to prescribed pressure. Next, measure the height of the vehicle at the points specified in the repair manual. If the height is lower than the prescribed value, the springs may be weakened by age.

- Check the installation of the springs

- Coil springs – the insulator should be installed properly and the spring should be firmly against the stopper.

- Leaf Spring

- Leaves should be checked for displacement in the lateral and longitudinal directions. Also, the spring clips should not be deformed.

- Check the U bolts and clips for looseness or damage

- Check the spring mounts for play in the longitudinal and lateral directions by prying carefully with a lever.

SUSPENSIONS LINKAGES

1. Pry the mounts of the body or frame side with a lever, checking for play in the longitudinal and lateral directions.

Front Suspension

- Upper Suspension Arms

- Lower Suspension Arms

- Strut Bars

- Stabilizer Bar

Rear Suspension

- Upper Control Arms

- Lower Control Arms

- Suspension Arms

- Lateral Control Arms

- Stabilizer Bar

If there is play, there could be improper tightening of the arm or rod, or the bushings could be worn

2. In the case of the strut type suspension, check for play in the upper support by pushing and pulling on the tires with both hands placed on top of the tire as shown here.

Check for damage to the installation area of the front suspension upper support.

3. Check the suspension arms for deformation.

4. Check the tightness of each arm and rod. Each arm and rod should be tightened to the prescribed tightening torque. Insufficient torque could throw the suspension geometry out of alignment, while excessive torque could cause deformation of the bushings and accelerate bushing aging and wear.

- Check the rivets and bolts of the spring brackets and suspension installation brackets for looseness. Also, make sure that the brackets are not cracked or damaged.

- Check ball joints for play.

SHOCK ABSORBERS

- Check for fluid leakage, deformation and damage.

- Check for cracks, damage or play in the installation areas.

- Check the operation of the shock absorbers. Move each corner of the vehicle up and down. The up-and-down motion should stop immediately after the hands released.

WHEEL ALIGNMENT

In order to confirm that the wheels are being held in the proper position and at the proper angle by the suspension, check the front wheel alignment and rear wheel tracking. If the alignment is improper, be sure to adjust so the values are within the prescribed range. If the adjustment cannot be made, check the suspension, steering system and axle.

- Front Wheel Alignment

- Rear Wheel Tracking

The left and right wheels should be installed so that they are symmetrical with respect to the vehicle’s centerline.

NOTES

- The optimum size and play or tire is specified for each vehicle model. Tires of a large size or play are in themselves heavy. Therefore their use will increase the unsprung weight and causes riding comfort to suffer. At the same time, since this will increase tire rigidity, shocks form the road surface will not be absorbed well by the tires and a hard ride will result.

- If the tire inflation pressure is higher than that specified the tires rigidity will increase, resulting in a hard ride. On the other hand, if the inflation pressure is too low, the ride will be too soft.

- Check the following items and if there are any defects, replace the shock absorber.

- Check to make sure that the shock absorbers of the correct specification for the car are being used.

- Check the operation of the shock absorbers. If a shock absorber’s damping force is insufficient, it will not be able to absorb the oscillation of the suspension springs, and the body will bob around. Particularly when driving on rough roads, the riding comfort will be extremely poor.

- Check to make sure the shock absorbers are installed without any play or looseness. At the same time, check for wear or cracks in the bushings.

- If a shock absorber is not installed correctly, its sliding resistance will become excessively large, it will not function properly and the necessary damping force will not be obtained.

- Check to make sure there is no fluid leakage.

- if the springs constant or installation of the springs is not correct, the car will sway badly on rutted, bumpy roads and riding comfort will suffer.

- Check to make sure that the springs with the correct specification of the car are installed.

- Check to make sure there is no weakening or damaged to the springs.

- Check to make sure that the installation of the springs is correct, and that there is no wear, weakening or cracks in the bushings, etc.

SELFCHECK	  	            :      5.3-2

QUALIFICATION		:	Automotive Servicing NC I

UNIT OF COMPETENCY	:	Perform Periodic Maintenance of Suspension

System

MODULE TITLE 		: 	Performing Periodic Maintenance of Suspension

System

LEARNING OUTCOME 3  	: 	Perform work to completion Information

INFORMATION SHEET		         :      5.3-3

QUALIFICATION		:	Automotive Servicing NC I

UNIT OF COMPETENCY	:	Perform Periodic Maintenance of Suspension

System

MODULE TITLE 		: 	Performing Periodic Maintenance of Suspension

System

LEARNING OUTCOME 3  	: 	Perform work to completion Information

Accomplish of job done/repair and 5s

Learning Objectives:

After reading this INFORMATION SHEET, YOU MUST be able to:

- Understand what is job done and 5’s

- Apply 5’s and job done on every work/task

- Appreciate the importance of job done and 5’s

Description

Job done is the declaration of the technician of what he/she had performed in the vehicle that you brought in the service center. It is usually located at the back of the repair order. It is consists of the time in and time out of the vehicle in the technician’s working bay, findings/recommendation about the vehicle, and the technician’s signature.

Repair Order is a purchase order for the repair of one or more items.  When purchasing a repair, repair IDs are used. These are the item or supplier number, service process, or service. Repair order category. Repair orders are defined with a specific order category in the purchase order type.

The 5S is the key point to make the environment of the workplace to work easily, quickly and certainly (safely.)

How does one ensure the quality of automotive service?

- Keep your workplace clean and orderly.

- Instead of trying to clean your workplace, make an effort not to dirty it in the first place.

| *1: | At Toyota the 4S concept is ordinarily used. The TEAM21 training uses an additional "SHITSUKE" to promote training under the 5S concept. |
| --- | --- |

1. SEIRI (sifting) This is a process for identifying whether certain items are needed, and those that are not needed are immediately discarded to use space effectively. - Organize and utilize all items, whether they are tools, parts, or pieces of information, in accordance with their necessity. - Designate an area in the workplace in which to place all unnecessary items. Collect unnecessary things in the working place, then scrap. - While it is important to carefully store things, it is just as important to - discard the items that are unnecessary.

|  | 2. SEITON (Sorting) This is a process for sorting tools and parts in an order that facilitates their use. - Place things that are rarely used in a separate area. - Place things that are used occasionally in your workplace. - Place things that are frequently used near you. |
| --- | --- |

- SEISO (Sweeping and Washing) 
This is a process for keeping everything in the workplace clean. Always keep that each function gives normally full play. And keep to use immediately when you want.

A dirty workplace is a reflection of your low self-esteem. Make a habit of keeping your workplace clean.

4. SEIKETSU (Spick and Span) 
This is a process for maintaining the SEIRI, SEITON, and SEISO conditions in an effort to prevent every possible problem from occurring. 
It is also a process for keeping your workplace clean by classifying everything and eliminating unnecessary items.

- Everything is a factor that contributes to the cleanliness of the workplace: the color, shape, and layout of all items, the illumination, ventilation, display shelves, and personal hygiene.

- If the working place becomes refreshing and bright environment, it is able to give a good atmosphere to the customer.

5. SHITSUKE (Self-Discipline) This process involves extensive training to make a person a proud Toyota employee. - SHITSUKE forms the basis of a culture and is a minimum requirement for ensuring harmony in society. - SHITSUKE is training for learning the rules. Through this training, the person will be worthy of being a Toyota employee. A person who is worthy of being a Toyota employee is one who is considerate of others, does not make others uncomfortable, and can do good deeds effortlessly.

APPLICATION OF 5’S

Before starting a job:

SEIRI- Prepare/borrow tools that are just needed in servicing suspension system.

SEITON- Arranged the tools according to necessity of usage. Remove other tools that are needed. For you to work in a speedy manner.

After finishing a task/job:

SEISO: Clean all the tools and equipment that  you used, to maintain its optimum function.

SEIKETSU- Keep your workplace clean. Classify everything in your working bay and remove all necessary items.

SHITSUKE- Be consistent. Repeat the process.

SELF-CHECK		         :      5.3-3

QUALIFICATION		:	Automotive Servicing NC I

UNIT OF COMPETENCY	:	Perform Periodic Maintenance of Suspension

System

MODULE TITLE 		: 	Performing Periodic Maintenance of Suspension

System

LEARNING OUTCOME 3  	: 	Perform work to completion Information

- Enumeration

Give the 5 meaning of 5’s.

- _______________________

- _______________________

- _______________________

- _______________________

- _______________________

II. Identification: Identify the following

- This is a process for identifying whether certain items are needed, and those that are not needed are immediately discarded to use space effectively.

- This is a process for sorting tools and parts in an order that facilitates their use.

- This is a process for keeping everything in the workplace clean. Always keep that each function gives normally full play. And keep to use immediately when you want.

- This is a process for maintaining the SEIRI, SEITON, and SEISO conditions in an effort to prevent every possible problem from occurring.

- This process involves extensive training to make a person a proud Toyota employee.

ANSWER KEY		         :      5.3-3

QUALIFICATION		:	Automotive Servicing NC I

UNIT OF COMPETENCY	:	Perform Periodic Maintenance of Suspension

System

MODULE TITLE 		: 	Performing Periodic Maintenance of Suspension

System

LEARNING OUTCOME 3  	: 	Perform work to completion Information

I. Enumeration

Give the meaning of 5’s

    - SEIRI

    - SEISO

    - SEITON

    - SEIKETSU

    - SHITSUKE

II. IDENTIFICATION: Identify the following

    - SEIRI

    - SEISO

    - SEITON

    - SEIKETSU

    - SHITSUKE